MEISTER CO., LTD. / GLOBAL PROJECT OFFICE

VIETNAM DIOXIN TECHNOLOGY ASSESSMENT PROPOSAL

Verification-led route for national, public and industrial projects

# 1. Executive proposal

MEISTER proposes a staged technology assessment for dioxin-contaminated soil and related POPs projects in Viet Nam. The proposal does not begin with a full-scale performance guarantee. It begins with document review, representative sampling, controlled testing, independent analysis and a jointly agreed pilot decision.

# 2. Why Viet Nam / why now

Viet Nam has identified remediation of dioxin-contaminated hotspots as a national environmental priority.

Official and UNDP materials show that contaminated-site projects require site-specific assessment, containment/remediation planning, safe land use and institutional coordination.

A new technology should therefore enter through a controlled evidence and regulatory-review pathway.

# 3. MEISTER technology routes

| Route | Target | Role |
| --- | --- | --- |
| Soil amendment / conditioning | Dioxin and PCB-contaminated soil | Evaluate adsorption, immobilization and related treatment under site-specific conditions. |
| A-MEISTER hydrothermal treatment | Organic matrices containing dioxin or other organics | Evaluate hydrolysis/decomposition, followed by analysis of solids, liquid, condensate and emissions. |
| Combined route | Mixed or difficult residues | Select sequential treatment only when analytical evidence supports it. |

# 4. Existing evidence presented for screening

WO2011074120A1 Example 1 reports dioxin WHO-TEQ (1998) changing from 81,122 to 543 pg-TEQ/g after six weeks; calculated reduction 99.33%.

A second disclosed example changes from 771.8 to 23.3 pg-TEQ/g; calculated reduction 96.98%.

These are individual examples, not an average, guaranteed result, or proof for another Vietnamese site.

# 5. Six-stage national-project pathway

| Stage | Deliverable | Decision |
| --- | --- | --- |
| 1 | NDA and existing-data review | Proceed to sampling plan |
| 2 | Representative samples and untreated controls | Test protocol approval |
| 3 | Paid laboratory treatability test | Technical response |
| 4 | Independent laboratory analysis and mass-balance review | Pilot eligibility |
| 5 | Pilot-scale field verification | Commercial and regulatory decision |
| 6 | Site-specific design, permit support and implementation plan | Contract / procurement |

# 6. Information requested

Site owner and competent authority

Location, history and land-use objective

Congener profile, TEQ basis, concentration range and analytical method

Soil type, moisture, organic matter, groundwater and estimated volume

Required cleanup criterion, schedule and residual-management route

Sampling, custody, import/export and laboratory requirements

# 7. Formal limitations

Chemical similarity between PCB and dioxins, and experience described in a PCB-related patent publication, do not independently prove performance for a new dioxin matrix. Final claims require representative testing, independent verification, emissions/wastewater review and acceptance by the competent Vietnamese authorities.

# 8. Official context sources

UNDP Viet Nam - Environmental remediation of dioxin contaminated hotspots: https://www.undp.org/vietnam/publications/environmental-remediation-dioxin-contaminated-hotspots-viet-nam-report-communication-component

Viet Nam national environmental strategy: https://vietnam.gov.vn/period-20112020/national-strategy-on-environment-protection-to-2020-with-visions-to-2030

Vietnam Academy of Science and Technology: https://vast.gov.vn/